2LINE Athene BOX

Optical telecommunication outlet / Customer line termination

Optical termination point or customer connection box in single and multiple family homes

- For fixation on the wall or on flush-mounted boxes

- As a conversion point from outdoor to in-house cable and as a subscriber connection socket in the home

- Holds up to 3 couplings and 12 splices

- Capacity of 3x SC- Pigtails or 6x LC-Pigtails

- Tube/cable entry and fixation up to max. Ø10mm

- Use of gas/water - stop

- Fibre routing according to bending radii with the possibility of depositing excess fibre lengths

- Cable entry from below or from the rear with sealing and integrated strain relief

- Closing self-locking, by means of Torx screw or seal against unauthorized opening

- Hinged, latching cover

- Exchangeable splice holder for storing shrink or crimp splice protection elements

- Can be combined with cable excess length box for use with assembled cables

Scope of delivery

- Splice protection holder 12x crimp or 6x shrink

- Cable entry sleeve with membrane (max.Ø7mm)

- Cable ties

- Screws for fastening the aramid yarn

- Locking Torx screws

- Assembly instruction

- Wall fixation material (Universal plugs and screws)

- Adapters and pigtails included (yellow) (optional)

- Customer logo (optional)

Varianten

Item: 2LINE Athene KÜB

Tests/Standards

Protection Index IP20

Impact resistant IK07

Self-extinguishing according to UL94 V0 and UV-resistant

Material

Plastic, signal white RAL 9003

Dimensions

HxWxD[mm]: 135x96x27

Article Code

2LINE Athene BOX

Article number

21073

Brand: ZweiCom-Hauff GmbH

Hauff-Technik GRIDCOM GmbH

Manufacturer information

Hauff-Technik GRIDCOM GmbH

Geiselroter Heidle 1

73494 Rosenberg, GERMANY

Telefon  +49 79 67 90 08 - 30

Fax +49 79 67 90 08 - 99