2LINE Network Distributor NVt 81

Fibre optic network distributor

Passive distribution point in FTTx fibre optic network

- Housing and base made of glass fibre reinforced 100% recyclable polycarbonate, coated in RAL 7038 with environmentally friendly paint

- Central grounding point

- Door with swivel lever, prepared for installation of a half profile cylinder

- Base with detachable front flap and C-rail for fixing the tube assemblies

- Interior fittings swivel frame with E&MMS cassette system and fibre management

- Inner rear panel for tube and cable management with cable excess length storage

- Fixation of incoming cables with central element retention

- Excess length storage of unused or uncut loose tubes (loop function)

- Excess length storage of outgoing cables

- Transfer and interception of cables in the splice area

- Protected guiding of the loose tubes to the splicing area

- Integration of splitters possible

Scope of delivery

- Splice cassette system 1-row with max. 60 locking places for up to 720 crimp splice or 360 shrink splice protection trays

- Sealed base plate with up to 2 feed-throughs for tube bundles and additional cable glands: outlet: 48x 7mm or 24x 10mm micro pipes; access: 3.5 - 20mm

- 1 pc. 2LINE NVt 81

Tests/Standards

Environmental protection class IP55

Impact resistance class IP54

Material

Glass fibre reinforced polycarbonate, aluminum, stainless steel

Dimensions

Housing HxWxD[mm]: 998x442x310; base HxWxD[mm]: 1060x442x284

Article Code

2LINE NVt 81

Brand: ZweiCom-Hauff GmbH

Hauff-Technik GRIDCOM GmbH

Manufacturer information

Hauff-Technik GRIDCOM GmbH

Geiselroter Heidle 1

73494 Rosenberg, GERMANY

Telefon  +49 79 67 90 08 - 30

Fax +49 79 67 90 08 - 99