2LINE Zelos+ BOX

Customer line termination

- Holds up to 12 couplings and 24 splices

- Storage capacity of 12x SC pigtails or 12x E2000 pigtails or 24x LC pigtails

- Cable/tube entry max. ø 13mm from below

- Possibility to use commercially available sealing elements in the housing

- Cover closure self-locking and grooved or Torx screw and sealable

- Hinged, latching cover

- Possibility to add on to increase capacity

- Fibre routing according to bending radii with the possibility of depositing excess fibre lengths

- Exchangeable splice holder for storing shrink or crimp splice protection elements

Scope of delivery

- Wall fixation material

- Optional: Adapters and pigtails included, customer logo, 2LINE ZAF, lock for protection against unauthorized access

Tests/Standards

Impact resistance class IP54 (for internal adapters)

Impact resistance class IP20 (for external adapters)

Fire resistance class UL94 V-0

Material

Plastic, color RAL9003

Dimensions

HxWxD[mm]: 300x194x64

Article Code

2LINE Zelos<sup>+</sup> BOX

Brand: ZweiCom-Hauff GmbH

Hauff-Technik GRIDCOM GmbH

Manufacturer information

Hauff-Technik GRIDCOM GmbH

Geiselroter Heidle 1

73494 Rosenberg, GERMANY

Telefon  +49 79 67 90 08 - 30

Fax +49 79 67 90 08 - 99