2LINE Optical Distribution Frame ODF 180

Optical distribution frame

- Option for symmetric-inverted construction

- Possible choice for disassembled or preassembled delivery

- Installation of 15" and 19" components

- Grounding option

- Height-adjustable frame base

- Fixation on the wall, on the ceiling, back to back or side by side

- Patch cable routing through cable duct on base frame or optionally on roof frame

- Top or bottom cabling entry options

- Cabling support plate

- Possibility for strain relief of the cable by means of a central element

- Modular assembly and expansion with splice/patch modules

- Fastening of the assemblies by means of pre-assembled threaded rails

- Orderly pick-up and fixation of the Miniflex tubes

- Uniform patch cable length in the ODF must be ensured under adherence to the minimum bending radii

- Possibility for modular system with up to 80 splitters

Scope of delivery

- 2LINE Central support element (optional)

- 2LINE Splice/Patch Modules (optional)

- 2LINE ODF door set, 2-piece with 3-point rod locking system, swivelling lever prepared for profile half cylinder (optional)

- 2LINE ODF side panel (optional)

- 2LINE Cable duct system at the top and bottom (optional)

- 2LINE Cover KAP (bundle loader protection) (optional)

- 2LINE Cover plate roof (optional)

- 2LINE Splitter holder and splitters (optional)

- 1 pc. 2LINE ODF 180

- 1 pc. Assembly instruction

- 2LINE Bracket clamps (optional)

Material

Stainless steel, steel galvanized and powder coated in RAL7035, plastic

Dimensions

HxWxD[mm]: 1800x900x300

Article Code

2LINE ODF 180

Brand: ZweiCom-Hauff GmbH

Hauff-Technik GRIDCOM GmbH

Manufacturer information

Hauff-Technik GRIDCOM GmbH

Geiselroter Heidle 1

73494 Rosenberg, GERMANY

Telefon  +49 79 67 90 08 - 30

Fax +49 79 67 90 08 - 99